- How is static generated?

Friction, pressure and separation are the major causes of static electricity. This process is called the Triboelectric Effect ("tribo" means rubbing). The triboelectric effect is responsible for the shock you receive from a car door after you slide out.

The magnitude of the static charge is determined by material composition, applied forces, separation rate, and relative humidity. These factors conspire to cause conveying mechanisms, and other systems, to act as static generators. Even slow-moving conveying mechanisms are capable of generating ample static charge to cause an ESD ( Electrostatic Discharge ) event. An uncontrolled ESD event can generate enough EMI to damage electronic components.

Environmental variables like relative humidity influence the level of electrostatic charges. When humidity is low, higher static charges are generated (see Table I). Static becomes more noticeable in the winter months, in dry climates, and in air conditioned environments. Increasing humidity to 60% limits static build-up because surface moisture on materials makes a good conductor. Unfortunately, 60% relative humidity is extremely uncomfortable, can cause equipment problems and introduce contaminants into your system.

Fortunately, static control devices can be located at critical points to control the ESD event by reducing the static charge to low levels. Static Control devices allow smoother operation of conveying mechanisms and eliminate electronic component damage caused by ESD events.

allow smoother operation of conveying mechanisms and eliminate electronic component damage caused by ESD events.

The Triboelectric effect is the major cause of static - the greater the force of friction or pressure, the more static will be created. Also, faster rates of separation cause more static.

| Table 1: Typical Electrostatic Voltage |  |  |
| --- | --- | --- |
|  | Electrostatic Voltages |  |
| Means of Static Generation | 10-20 percent Relative Humidity | 65-90 percent Relative Humidity |
| Walking across carpet | 35,000 | 1,500 |
| Walking over vinyl floor | 12,000 | 250 |
| Worker at bench | 6,000 | 100 |
| Vinyl envelopes for work instructions | 7,000 | 600 |
| Common poly bag picked up from bench | 20,000 | 1,200 |